MINIMUM EQUIPMENT LIST

##### N727RH

## GENERAL

This Minimum Equipment List is to be used prior to flight to determine if this aircraft is legal to fly with one or more components inoperative. Multiple inoperative components are permitted unless specifically prohibited in the text. Any component not included in this list must be operative for flight.

Once in flight, this MEL may be used as a reference, but is not controlling.

There are no specific time limits for repair on inoperative items, i.e. an item listed in this MEL may be inoperative indefinitely as long as flight is permitted and conducted in accordance with the appropriate MEL section.

##### MINIMUM EQUIPMENT LIST

##### N727RH

## CONTENTS

## COMPONENT INDEX:    Alphabetical listing of MEL items

## SECTION A:        Required to be operative for ALL FLIGHTS

## SECTION B:	    May be inoperative For ANY FLIGHT

## SECTION C:	    Required to be operative for NIGHT FLIGHT

## SECTION D:        Required for flight into Class B airspace

## SECTION E:        Required to be operative for INSTRUMENT FLIGHT

(May be inoperative for VFR FLIGHT)

##### MINIMUM EQUIPMENT LIST

##### N727RH

## COMPONENT INDEX

Component			Listed in SECTION:

Airspeed Indicator					A

Altimeter						A

Ammeter						B

Artifical Horizon					E

Attitude Indicator					E

CD Player					B

Clock						E

Comm Radio					D, E

Cylinder Head Temperature Gauge			B

Defroster						B

DG						E

DME 						E

EGT Gauge					B

Electric Fuel Pump					A

ELT						A

Encoding Altimeter					D

Exhaust Gas Temperature Gauge			B

Fire extinguisher					B

Fuel Flow Gauge					B

Fuel Pump - Electric				A

Fuel Quantity Indicator - Main Tank			A

Fuel Quantity Indicator - Wing Tanks			B

Generator					E

Glideslope Receiver and Indicator 			E

GPS Power jack					B

Heading Indicator					E

Heater						B

Hobbs Meter					B

Intercom						B

Iinstrument Panel Lights				C

Landing Light					B

Magnetic Compass					A

Magnetos						A

Main Tank Fuel Quantity Gauge			A

Manifold Pressure Gauge				B

Marker Beacons 					E

##### MINIMUM EQUIPMENT LIST

##### N727RH

## COMPONENT INDEX (con’t)

Component			Listed in SECTION:

Nav Lights					C

Oil Pressure Gauge					A

Oil Temperature Gauge				A

Outside Air Temperature Gauge			B

Pitot Heat						B

Position Lights					C

Strobes						C

Suction Gauge					E

Tachometer					A

Taxi Light						B

Transfer Pump					B

Transponder 					D, E

Turn and Bank Indicator				E

Turn Coordinator					E

Vacuum Gauge					E

Vacuum Pump					E

Vertical Speed Indicator				B

Voltmeter 					B

VOR Receiver and Indicator 				E

Wing Tank Fuel Quantity Gauges			B

# MINIMUM EQUIPMENT LIST

##### N727RH

## SECTION A:

Required to be operative for ALL FLIGHTS:

Airspeed Indicator

Altimeter

Magnetic Compass

Tachometer

Oil Pressure Gauge

Oil Temperature Gauge

Magnetos  (both must be operational)

Electric Fuel Pump

ELT  ** Except as provided by FAR 91.207 (e)(10) **

Main Tank Fuel Quantity Indicator    ** Unless **:

1)	Fuel Quantity in Main Tank is determined for Weight & 			Balance purposes

2)	Both Wing Tank Fuel Quantity Gauges are operative

3)	Main Tank Fuel Selector is positioned to "OFF"

4)	Wing Tank Routing Selector is positioned to "DIRECT TO 	ENGINE"

##### MINIMUM EQUIPMENT LIST

##### N727RH

## SECTION B:

May Be Inoperative For ANY FLIGHT:

Manifold Pressure Gauge

Cylinder Head Temperature Gauge

Fuel Flow Gauge

EGT Gauge

Ammeter OR Voltmeter  ** One must be operative **

Vertical Speed Indicator

Outside Air Temperature Gauge

GPS Power jack

CD Player

Hobbs Meter

Pitot Heat

Heater, defroster

Intercom

Fire extinguisher

Landing and Taxi lights

Wing Tank Fuel Quantity Gauges

** Provided **:

1) Fuel quantity in inoperative wing tank(s) is 			determined for Weight & Balance purposes.

(Tank may be filled, transferred or drained.)

and 2) Inoperative wing tank fuel is not selected for use.

Transfer Pump

** Provided**:

1) When Wing Fuel is desired,

WING FUEL ROUTING selector is positioned to

“DIRECT TO ENGINE”

##### MINIMUM EQUIPMENT LIST

##### N727RH

Sections C, D and E include components that are required in addition to those items listed in SECTION A.

## SECTION C:

Required to be operative for NIGHT FLIGHT:

Nav lights (Position lights)

Strobes

Instrument panel lights  (Sufficient lighting must be available)

## SECTION D:

Required for flight into Class B airspace:

Transponder             (also required under Class B)

Encoding Altimeter     (also required under Class B)

Comm radio

## SECTION E:

Required to be operative for INSTRUMENT FLIGHT,

(May be inoperative for VFR FLIGHT):

Attitude Indicator  / Artifical Horizon

Heading Indicator / DG

Suction Gauge / Vacuum Gauge

Vacuum Pump

Turn Coordinator / Turn and Bank

Comm Radio

Clock

Generator

##### MINIMUM EQUIPMENT LIST

##### N727RH

May be required to be operative for INSTRUMENT FLIGHT, as required navigational equipment appropriate to the ground facilities to be used

(FAR 91.205 (d)):

Transponder

VOR Receiver and Indicator

Glideslope Receiver and Indicator

Marker Beacons

DME